Lecture Note for Numerical Analysis (12) Fourier Approximation

- Introduction to the Periodic Function

- A periodic function f(t) is one for which

where T is a constant called the period that is the smallest value for which this equation holds.

(Examples)

(a) square wave

(b) saw-tooth wave

(c) non-ideal wave

(d) wave contaminated by noise

- Sinusoidal function

- Any waveform that can be described as a sine or cosine is called sinusoid:

4-parameters

-: mean value

-: amplitude ()

-angular frequency

-: phase angle

- Definition of period:

- Definition of the phase angle

- Definition of Complex Exponent

- Example: Least Squares Approximation with Sinusoidal Functions

- Given n data points :

- Regression:  find a curve fitting best to the points

optimality condition

- For a special case with equal spacing in  with the period such as

where

If is a integer multiple of such as for some positive integer

,then

Therefore,  and

Furthermore,

If we repeat above procedure for other terms using

Finally, we can get

- Continuous Fourier Series

- Fourier Series Expansion of a Continuous Function with the period

or more concisely

: is called as the fundamental frequency

: 1st harmonic, 1st harmonic amplitude

: 2nd harmonic, 2nd harmonic amplitude

: 3rd harmonic, 3rd harmonic amplitude

: 4th harmonic, 4th harmonic amplitude

etc

- Important properties in the sinusoidal expansion of a continuous function with the period

For two positive integers

Therefore,

- Calculation of Fourier coefficients

(i)

(i)

Therefore,

for

- Complex Form of the Fourier Series

- Using the following relations

Let’s define

Then

k-th harmonic amplitude	=

k-th harmonic phase	=

- Frequency Domain Concept Compared to Time Domain Record

- Time domain Plot

- x-axis: time

- y-axis: time varying amplitude

- Frequency domain Plot

- x-axis: frequency

- y-axis: amplitude at each frequency

phase angle at each frequency

- Fourier Integral and Inverse Transform

- Fourier Integral

- Inverse Fourier Transform

- Discrete Fourier Transform (DFT)

- In engineering, functions are often represented by finite sets of discrete values and data is often collected in or converted to such a discrete format.

- An interval from 0 to T can be divided into N equispaced subintervals with widths of

- DFT formula

- Inverse DFT formula

- Fast Fourier Transform (FFT)

- FFT is an algorithm that has been developed to compute the DFT in an extremely economical (fast) fashion by using the results of previous computations to reduce the number of operations.

- FFT exploits the periodicity and symmetry of trigonometric functions to compute the transform with approximately N log2 N operations. Thus for N=50 samples, the FFT is 10 times faster than the standard DFT. For N=1000, it is about 100 times faster.

(8-1) Sande-Tukey Algorithm for the 1st stage

- Normalize time with  as

- Let’s define a complex-valued weight function as

Therefore,

for odd number 2k+1

for even number 2k

Let’s define

Then the computation of can be written as

And the resultant computing time has been halved.

(8-2) Sande-Tukey Algorithm for the 2nd stage

Using

- Comparison of the Computational Time between DFT and FFT

Appendix: Important topics in Discrete Fourier Transform

1. Aliasing : Folding the frequency contents with varying sampling intervals

Function: y(t) = cos(60*t);

Sampling intervals: Dt= 0.001 sec, 0.01 sec, 0.05 sec, 0.1 sec

One would expect that if the signal has significant variation then Ts must be small enough to provide an accurate approximation of the signal x(t). Significant signal variation usually implies that high frequency components are present in the signal. It could therefore be inferred that the higher the frequency of the components present in the signal, the higher the sampling rate should be. If the sampling rate is not high enough to sample the signal correctly then a phenomenon called aliasing occurs.

The term aliasing refers to the distortion that occurs when a continuous time signal has frequencies larger than half of the sampling rate. The process of aliasing describes the phenomenon in which components of the signal at high frequencies are mistaken for components at lower frequencies.

The Nyquist Sampling Theorem states that to avoid aliasing occuring in the sampling of a signal the samping rate should be greater than or equal to twice the highest frequency present in the signal. This is refered to as the Nyquist sampling rate.

(a) Sampled Data

(b) Frequency contents

(c) Matlab Program

%  aliasing.m

%

%  This m-file illustrates the effects of aliasing in time and frequency domains

%

% Four time bases, at four sampling rates.

t1 = 0:.001:8;

t2 = 0:.01:8;

t3 = 0:.05:8;

t4 = 0:.1:8;

% Four samples of a cosine at 60 rad/sec

s1 = cos(60*t1);

s2 = cos(60*t2);

s3 = cos(60*t3);

s4 = cos(60*t4);

% Plots of the four samples

subplot(221);

plot(t1,s1);

axis([0 1 -1 1]);

title('.001 s sampling rate');

subplot(222);

plot(t2,s2);

axis([0 1 -1 1]);

title('.01 s sampling rate');

subplot(223);

plot(t3,s3);

axis([0 1 -1 1]);

title('.05 s sampling rate');

subplot(224);

plot(t4,s4);

axis([0 1 -1 1]);

title('.1 s sampling rate');

clc

fprintf('Results show the effect of different sampling frequencies\n');

fprintf('and aliasing.\n\nThe sampled function is cos(omega*t)\n');

fprintf('where omega is 60 rad/sec\n\n');

fprintf('Note that if the time between samples is more than half the period of\n');

fprintf('the sampled function then an error occurs in the sampling.\n');

fprintf('This error is called aliasing.\n\n');

fprintf('In other words, to avoid aliasing when sampling a signal,\n');

fprintf('the sampling frequency must be greater than twice the highest\n');

fprintf('frequency component in the signal. This is the Nyquist Sampling Theorem.\n\n');

fprintf('Press any key to continue.\n\n\n\n');

pause

clf

clc

fprintf('Aliasing effects are also evident in the frequency spectrum\n\n');

fprintf('If the sampling frequency is less than the Nyquist frequency then the\n');

fprintf('frequency components higher than the Nyquist frequency appear erroneously\n');

fprintf('as lower frequencies.\n\n');

fprintf('The higher frequencies "fold over" around the Nyquist frequency\n\n\n');

fprintf('This series of spectrum plots illustrate the effect of changing the\n');

fprintf('sampling rate on the frequency domain representation of the signal\n\n');

S1 = fftshift(abs(fft(s1,1024)));

S2 = fftshift(abs(fft(s2,1024)));

S3 = fftshift(abs(fft(s3,1024)));

S4 = fftshift(abs(fft(s4,1024)));

w1 = -500:1000/1024:500-1/1024;

plot(w1,S1);

fprintf('Frequency spectrum: First sampling rate.\n');

fprintf('Peak is at 60 rad/sec.\n\n');

fprintf('Press any key to continue.\n\n');

pause

w2 = -50:100/1024:50-1/1024;

plot(w2,S2);

fprintf('Frequency spectrum: Sampling rate halved.\n');

fprintf('Peak is still at 60 rad/sec.\n\n');

fprintf('Press any key to continue.\n\n');

pause

w3 = -25:50/1024:25-1/1024;

plot(w3,S3);

fprintf('Frequency spectrum: Sampling rate again halved.\n');

fprintf('Peak is still at 60 rad/sec, but it''s close to the edge.\n\n');

fprintf('Press any key to continue.\n\n');

pause

w4 = -5:10/1024:5-1/1024;

plot(w4,S4);

fprintf('Frequency spectrum: Sampling rate again halved.\n');

fprintf('60 rad/sec peak now appears to be much lower. Aliasing has occurred.\n\n');

2. Gibbs Phenomena for Partial Fourier Transform for f(t) = 100 ( 0<t<3.14159 (pi))

With varying number of harmonics

One shortcoming of Fourier series today known as the Gibbs phenomenon was first observed by H. Wilbraham in 1848 and then analyzed in detail by Josiah W. Gibbs (1839-1903)

(a)  Phenomena

N = 5                             N =10                      N = 15

N = 50                        N = 100                   N = 500

Even with large number of harmonic terms, Oscillatory behavior never disappears

Window concept can be used to reduced high frequency content of oscillations

(b)  Matlab Program

%function gibbs(n)

% An illustration of Gibbs phenomenon.

% This function computes the partial Fourier series sum of a square wave, to illustrate the peaks that

%  occur  at jump discontinuities when using the Fourier series.

%

% The function plots a square wave. and asks the user for the number of terms to use in the Fourier

%  series sum. The partial Fourier series approximation is then superimposed upon the square wave.

%

% variable n is the number of terms to use in the partial sum.

% create the square wave

dt = 0.001;

t = 0:dt:pi;

f = 100 * ones(size(t));

f(1) = 0;

f(length(f)) = 0;

%create the partial Fourier series approximation

s = zeros(size(t));

for i = 1:n

s = s + 1/(2*i - 1)*sin((2*i - 1) * pi * t / pi);

end

s = 400 / pi * s;

%plot the approximation and the square wave

plot(t, s, '-r', t, f, '--b');grid

title('Gibbs Phenomenon');

Useful Information at signal processing toolbox at MATLAB

>>help signal

>> help window

>> help fdesing

See also bartlett, barthannwin, blackman, blackmanharris, bohmanwin,

chebwin, gausswin, hamming, hann, kaiser, nuttallwin, parzenwin,

rectwin, triang, tukeywin, wintool.

============================================

>>d = fdesign.nyquist(5,'n',150); hd = window(d,hamming(151)); fvtool(hd)

3. FFT example of a signal: MATLAB Program (for f(t) = cos(3*2*pi*t) + cos(1*2*pi*t);

% fft_ex.m

%

% This is an example of computing the FFT of  the addition of two cos functions.

%

clc

fprintf('**************************************************************\n\n');

fprintf('This file illustrates how to perform the FFT on a time\n');

fprintf('signal using MATLAB\n\n\n');

fprintf('In this case, the time signal is the sum of two cosines\n\n');

echo on

% Sample at 10 Hz, and use a 500 second long sample -> 5001 samples

t = 0:.1:500;

% Calculate the sum of the two cosines

x = cos(3*2*pi*t) + cos(1*2*pi*t);

% Take an 8192-point FFT (power of 2 greater than 5000)

f = fft(x,8192);

% Construct a frequency axis

Freq = -5:10/8192:5-1/8192;

% Plot frequency, magnitude.  fftshift centers around zero.

plot(Freq, abs(fftshift(f)));

echo off